RAJARATA UNIVERSITY OF SRI LANKA
Department of Computing

### ICT2403 – Graphics and Image Processing

## Submission 03 – Frame Enhancement Method

Course Code: ICT2403
Course Title: Graphics and Image Processing

Assignment Title: Frame Enhancement Method for Road Damage Detection

### Submitted By

| Member Name | Registration Number | Index Number |
| --- | --- | --- |
| Mr.H.M.T.D.M.Amarasingha | ICT/2023/058 | 6046 |
| Mr.H.M.N.R.B.Herath | ICT/2023/059 | 6047 |
| Mr.D.M.R.V.Dissanayaka | ICT/2023/060 | 6048 |
| Mr.D.S.M.D.B.Subasingha | ICT/2023/083 | 6065 |

Submitted To:
Mr.K.A.S.H.Kularathna

Department of Computing
Rajarata University of Sri Lanka

- Introduction to Frame Enhancement

Image enhancement is and is a essential preprocessing step in a road damage detection system. Raw video frames may contain various types of noises that reduce image clarity and can affect crack detection.

One common type of noise is sensor noise. These noises can cause by Low-light conditions, camera sensor limitations and electronic interference. It causes small random intensity variations and grainy texture.

Video frames also can capture salt and pepper noise caused by transmission errors, sudden bit corruption or hardware malfunctions. It appears as random white and black pixels.

Another possible noise is Gaussian noise can have caused by electronic circuit fluctuation or sensor thermal noise. It appears as grainy effect.

Frame enhancement techniques such as contrast stretching, histogram equalization and noise filtering help to prevent these issues. Noise reduction smooths unwanted intensity variants. Enhancement provides clear structural information and improve the accuracy.

- Identification of Frame Degradations

The extracted frames from the road video were analyzed to identify quality degradations that may affect crack detection accuracy. Visual inspection , histogram analysis, and standard deviation measurements were used to detect and evaluate these degradations.

Noise Analysis

Sensor Noise (Mobile Camera Noise)

Sensor Noise is visible in the extracted frames as small random intensity variations . This noise is primarily caused by mobile camera sensor limitations, low light conditions.

Low-Light Noise Effect

Low-light were detected by observing darker regions in the frames and analyzing histogram distributions

Illumination Variations

Uneven illumination affects crack detection because segmentation algorithms rely on intensity differences. When lighting is inconsistent, cracks may either merge with shadows or become less visible

Motion Blur and Camera Instability

This degradation was identified by visually examining the sharpness of crack edges and comparing frame clarity across the video sequence.

- Pre-processing of Video Frames

A video is a sequence of pixel frames displayed over time. To enable image processing and analysis The video needs to convert into frames. It enables further image processing and analysis.

Frame Extraction Method

Frame extraction was performed using the OpenCV library.

Videos were loaded using cv2.VideoCapture()

Frames were read sequentially in a loop

Each frames were using read() function

Frame Format

Extracted frames were saved in JPG format. The JPG format provides efficient storage with image quality suitable for processing multiple frames.

Frame Selection Strategy

Every 30th frame was extracted and saved for analyzing. This approach reduces computational load while helping to capture sufficient structural information for crack analysis.

- Proposed Image Enhancement Pipeline

Purpose: Expand intensity range 0 - 255

Purpose: Remove Gaussian Noise

Kernel Size: 5 * 5

Purpose: Reduce Complexity

Formula: 0.299 * R + 0.587 * G + 0.114 * B

Frame extraction and noise analysis - https://github.com/dhananjaya-subasingha/-Frame-Enhancement-Method

Contrast enhancement and illumination correction - https://github.com/virajDissanayaka/ICT2403-Graphics-and-Image-Processing.git

Sharpening and edge preservation - https://github.com/NirmalHerath-cmd/ICT2403---Graphics-and-image-processing.git

Pipeline integration and testing - https://github.com/thrusha506/ICT2403-Graphics-and-Image-Processing.git

Videos https://drive.google.com/drive/folders/1ZVdq1rUqeI_5qkQjhwKIiS1-YygAGnsm?usp=drive_link

Grayscale Conversation

The purpose of grayscale conversation is to reduce computational complexity while processing the frames. Since road cracks are mainly depending on intensity variations grayscale is more sufficient than color frames.

Formula

Grayscale = 0.299 * R + 0.587 * G + 0.114 * B

Code statement

gray_image = cv2.cvtColor(frame,cv2.COLOR_BGR2RGB)

Noise Reduction

Noise is a visual distortion in digital images that causes degradation. Gaussian noise arises due t electronic circuited noises and sensor noisiness.

Formula

G(x,y) = f(x,y) + n(x,y)

Code Statement

gaussian = cv2.GaussianBlur(image , (5,5) , 0)

Contrast Enhancement

Road cracks has low-intensity regions that blends with road surface. Contrast stretching can expand hee intensity range and improve visibility.

Formula

S = { a + ( r – r min ) ( b – a ) } / ( r max –  r min )

r = Original pixel value

a and b = Desired minimum and maximum values

Code Statement

def contrast_stretch(image):

min_val = np.min(image)

max_val = np.max(image)

if max_val == min_val:

return image.copy()

stretched = (image - min_val) * (255 / (max_val - min_val))

return stretched.astype(np.uint8)

Graysale						Sharpened

Before edged							after edged

- Experimental Setup and Results

Test Data Preparation

The test dataset was prepared from recorded road inspection videos. This video was processed using OpenCV and frames were extracted using a frame sampling strategy.

Frame Selection Criteria

Every 30th frame was saved in JPG format to reduce redundancy while preserving structural variation in road surface conditions.

Image Format and Resolution

Format: JPG

Resolution : Original video resolution

Color Mode: Converted to grayscale

Evaluation Metrics Selected

Laplacian Variance

This operator used to measure image   sharpness. Laplacian can detect regions of intensity changes.

def measure_blur(image):

laplacian = cv2.Laplacian(image, cv2.CV_64F)

return laplacian.var()

Canny Edge detection analysis

Canny edge detection was applied before and after sharpening to visually compare enhancement.

edges_before = cv2.Canny(image, 100, 200)

edges_after = cv2.Canny(sharpened, 100, 200)

Testing the enhanced pipeline

The pipeline was tested using extracted videoframes from the dataset. Each frames was sequentially processed through following stages

Contrast enhancement

Sharpening

Edge visualization

Histogram

Parameter Settings

Gaussian Blur

Kernel size 5 * 5

Reduce high-quality frequency and prevent excessive edge amplifications

Sharpening

Original Image weight: 1.5

Blurred image weight: -0.5

Gamma: 0

Enhance edge boundaries

Histogram

Range 0-255

Intensity 256

Spreads pixel intensity values

Canny Edge Detection

Lower threshold: 100

Upper threshold: 200

Results

- Recommendation

Final Recommendation based on result

Based on results, the image enhancement pipeline successfully improved the quality of video frames. The results obtained from the standard deviation measurements indicate that noise levels were reduced after applying Gaussian and Median filtering. Sharpening improved the visibility of structural edges within frames. The histogram comparisons and visual inspection of the enhanced frames also showed improved intensity distribution.

Critical Analysis of Enhancement Techniques

Gaussian filter – Reduce random noise present in the extracted frames

Median Filter – Reduce impulse noise such as salt and pepper noises.

Sharpening – Enhance the structural details of the frames.

Limitations of the Proposed Enhancement Method

Strong shadows sometimes affected the visibility of road cracks.

Extreme lightning reduces the effectiveness of enhanced techniques.

Motion blur caused by camera movements.

Impact on Future Segmentation Accuracy

The enhanced frames provide better input data for the segmentation stage of the road damage detection system

Noise reduction helps prevent segmentation algorithms from detecting false edges

Cracks and potholes more distinguishable from the surrounding road surface.

Edge sharpening improves the visibility of crack boundaries.